Task 6

Contents

Contents	2

List of Figures	3

No table of figures entries found.	3

List of Tables	4

1	General	5

1.1	Approvals and Dates	5

1.2	Change Record	5

1.3	Acronyms	6

1.4	References	7

2	Introduction	8

3	Main Heading	9

3.1	Second Level Heading	9

3.1.1	Third Level Heading	9

List of Figures

Figure 1 – Figure Title	9

List of Tables

Table 1 – Example Table	9

# General

## Approvals and Dates

|  | Approval Date |
| --- | --- |
| A. E. Jones |  |

## Change Record

| Date | Version |  | Author | Description of Changes |
| --- | --- | --- | --- | --- |
| Date | 0.01 |  |  | Initial Draft |

## Acronyms

| EPC | Evolved Packet Core |
| --- | --- |
| LTE | Long-Term Evolution |

## References

- example reference

# Introduction

## Uplink Reference Signal

DeModulation RS (DM_RS) is associated with the transmission of uplink data on PUSCH or PUCCH. The configuration of a PSUCH is shown in Figure 1, the DM_RS of a give UE occupies the same bandwidth (Number of resource blocks) as the PUCCH/PUSCH data transmission.

| 12 Subcarriers = 180KHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7 Symbols = 0.5ms |  |  |  |  |  |  |

| 12 Subcarriers = 180KHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6 Symbols = 0.5ms |  |  |  |  |  |

Figure 1- Normal and Extended Cyclic prefix for configuration of DM_RS PUSCH

The desirable characteristics of a reference signal:

- Constant amplitude in the frequency domain for equal excitation of all the allocated subcarriers for unbiased channel estimators

- Low cubic metric in the time domain, at worst no higher than that of the data transmission. It can be beneficial if lower cubic metric is achieved for a reference signal than the data, enabling the transmission power of the reference signal to be boosted at the cell edge.

- Good autocorrelation properties for accurate channel estimates as shown in task 4

- Good cross-correlation properties between difference reference signals to reduce interference from reference signals transmitted on the same resource.

# Zadoff-Chu Sequence

The zadoff-chu sequences are non-binary sequence unlike the m-sequence seen in task 4, it satisfies a constant amplitude zero autocorrelation (CAZAC) property where the complex signals are of the for ejαk. The ZC sequence of odd length NZC is defined as:

|  | xun=e-j2πunn+12+lnNZC |  |
| --- | --- | --- |

Where u ϵ 1,…NZC-1 is the ZC root index, n=0,1,2,…,NZC-1 and for an LTE system l=0. The ZC sequence has a constant amplitude, where the Discrete Fourier Transform of ZC also has a constant amplitude, this limits the Peak to Average Power Ratio and can be seen in subplots one and two of Figure 2. In addition the cyclic correlation properties of the ZC sequence are ideal i.e. the correlation with its circular shifted version of itself provides zero for all index points other than that of full correlation where the magnitude at this point it is equal to NZC which can be seen in subplot three. Similarly to task 4 the point of interested is when the received reference signal is correlated with a local replica of the reference signal, therefore depicting the propagation delay and channel properties. Figure 2 show a ZC sequence where the root is chosen to be 25, 29 and 34, it is clear that ZC sequence circulates around the unit power circle, where autocorrelation properties for each ZC sequence are ideal.

Figure 2 - Zadoff-Chu Sequences

The two main benefits for using this sequence above that of the M-sequence shown in task 4 is that it allows multiple orthogonal sequences to be defined for the same ZC sequence and the unit power qualities are achieved in both the time and frequency domain reducing the PAPR. For instance if the periodic autocorrelation of ZC shows a peak at index zero shown Figure 2 subplot 3, then the periodic correlation of the same sequence against its cyclic shifted replica where the peak is defined at Ncs the number of samples of the cyclic shift i.e. in Figure 3 Ncs=4. It can be seen that there is zero correlation between the two index points shown in subplot one and two, therefore if zero correlation zone ZCZ is chosen such that the largest delay spread is smaller than the duration of ZCZ then the two sequences are orthogonal for all transmission within the delay spread.

Figure 3-Cyclic Shifted ZC

A useful aspect of the zadoff-chu sequence especial when considering LTE systems is that the zadoff-chu sequence can be generated directly in the frequency domain where the DFT of a ZC sequence xun defined in Equation 1. is a weighted cyclically shifted ZC sequence Xwk where:

|  | w= -1umod NZC |  |
| --- | --- | --- |

## Demodulation Reference Signals

The demodulation Reference signals are based around the Zadoff-Chu sequence where the Zadoff-Chu sequences exhibits 0 dB cubic metric, ideal cyclic autocorrelation and optimal cross correlation. The cross correlation properties ensure that an interfering signal is spread evenly in the time domain after time domain correlation of the received signal with the local replica, hence provides a more reliable detection of the channel taps. However the cubic metric for a ZC sequence is degraded due to the Nyquist sampling rate, as the unused guard subcarriers at each end of the sequence. This results in the ZC sequence be effectively oversampled in the time domain.

The length of the reference signal Np is defined as:

|  | Np=MScRs=m.NscRb 1≤m≤NRBUL |  |
| --- | --- | --- |

Where NscRb is the number of subcarriers per resource block, NRBUL is the maximum number of uplink resource blocks. Sequences generated for three or more resource blocks provide at least 30 sequences with a cubic metric smaller than or close to that of QPSK. However this is not the case for less than three resource blocks, in this case for one and two resource blocks there are six or twelve sequences respectively. Therefore 3GPP have defined a special set of sequence to provide as many sequences as three resource blocks case. In both these cases a Zadoff-Chu sequences is not used, rather a specially designed set of QPSK sequences are defined obtaining the required characteristics for a reference signal. Figure 4 shows the time and frequency domain response for both special cases. It is clear that the frequency domain response no longer obtains a perfect unit magnitude and therefore the PAPR would will be increased as a result, This quality is also seen in the large sequence sizes.

Figure 4-Special Case sequences Nrb and 2Nrb where u ranges from 0,1,2,..,29

For the case where MScRs≤2NscRb the base sequence is given by :

|  | ru,vn=ejφnπ/4 ,0≤n≤MScRs-1 |  |
| --- | --- | --- |

where the base sequences ru,vn are divided into groups, where u is base sequence number ranging for 0 to 29 and v is the base sequence number within the group. φn is defined in 3GPP document 36.211 Table 5.5.1.2-1 and Table 5.5.1.2-2.

For the case where MScRs≥3NscRb the base sequence is given by:

|  | ru,vn=xqn mod NZCRS ,0≤n≤MScRs-1 |  |
| --- | --- | --- |

where NZCRS is the largest prime number smaller than MScRs

|  | ru,vn=xqn mod NZCRS ,0≤n≤MScRs-1 |  |
| --- | --- | --- |

The zadoff-Chu sequence xq defined as:

|  | xqm=e-jπqmm+1NZCRS ,0≤m≤MScRs-1 |  |
| --- | --- | --- |

where m is obtained from Equation 6 m=n mod NZCRS. The ZC sequence root is defined as:

|  | q=q+12+v.-12q q=NZCRS.u+1/31 |  |
| --- | --- | --- |

finally v defined as:

|  | v=0,1 for MScRs≥72 0 otherwise |  |
| --- | --- | --- |

Figure 5 shows four generated reference signals with varying base sequence numbers. It is clear that the ideal characteristics of the reference signal have been obtained, where the magnitude is constant for all base sequences, shown in subplot 1. The phase is the factor which varies and is shown in subplot 2 and the final subplot plot shows the cross correlation between all the reference signals and the first reference signal. Hence the first plot shows Ideal auto correlation, where the remaining plot show little correlation between itself and the first reference signal. Demonstrating that each base sequence number u provides a reference signal which has minimum cross correlation between other base sequence number.

Figure 5-Generated Reference Signals

The finalised reference signal for both cases is given as:

|  | ru,vαn=ejαnru,vn, 0≤n≤MScRs-1 |  |
| --- | --- | --- |

where alpha is the cyclic shift applied to the base sequence where for PUSCH there are 12 difference cyclic shifts available, for a sounding reference signal that are 8:

|  | α=2πgNumber of Cyclic Shifts |  |
| --- | --- | --- |

Where g is the cyclic shift index and ranges from 0 to 11 and 0 to 7 for PUSH and SRS respectively. The number of cyclic shifts is determined in the same manner as the zero correlation zone shown in section 2.7.1. Where alpha represents a phase ramp in the frequency domain and therefore a cyclic shift in the time domain, it can be seen from Equation 11 that the phase ramp is between 0 to a fraction of 360 degrees as to apply a cyclic shift of 360 degrees is the same as apply a cyclic shift of 0 degrees due to full rotation. The reason that alpha is implemented is in the case of MIMO multiple UE's can be assigned to the same resource block, it is undesirable to uses the exact same reference signal for each UE as the reference signal can interfere with each. Therefore alpha enables multiple orthogonal sequences to be generated, rather than using different sequence for different UE's in the same resource block  which is not ideal due to the no zero cross correlation between the base sequence which can degrade the channel estimated obtained by the eNodeB.

Figure 6 demonstrates how multiple UE's can use the same reference if a cyclic shift is applied to obtain multiple orthogonal sequences where the number of orthogonal sequences is limited by the maximum delay spread of the channel and by the number of subcarriers in a resource block. Below is the theoretical number of cyclic shift that can be obtained:

|  | Available Cyclic Shifts= TTChannel |  |
| --- | --- | --- |

However this is not practical as the number cyclic shifts is required to be an integer multiple of the number of subcarriers in a resources block and hence in PUCCH there are 12 cyclic shifts available, meaning that the orthogonal qualities are maintained if the delay spread is less than 5.56 us which is greater than the cyclic prefix appended to the OFDM symbol. If the available Cyclic shifts are 12 then the eNodeB receiver can distinguish between 12 UE's by correlating the received composite signal from the different transmitters occupying the same resource block with the base sequence, the channel estimates for each sequence is separable in the time domain. This can be seen from Figure 6 where it is clear that each shifted reference signal delayed.  This was also shown in task 4 where a cyclic shifted signal is the same as applying a phase ramp in the frequency domain. The maximum delay spread that the PUSH and SRS  reference signal can cope with is 5.56 us and 8.33 us respectively.

Figure 6-Cyclic Shifted Reference Signal

# Peak to Mean Power Ratio

Assessing the simplified version of the OFDM symbol for a 5MHz channel where there are 300 subcarriers and the sample size is 512. From task 5 we know that an OFDM symbol is the sum of sinusoidal with a frequency separation of fkwhere fk is frequency of the subcarrier and hence we can write a simplified OFDM Symbol as:

|  | OFDM(t)=k=0N-1Xkejθkej2πfkt |  |
| --- | --- | --- |

Where  Xkejθk is the modulated data, Figure 6 show subcarriers 1 through to 3 being set to unit where the time domain response is the summation of the waveforms as shown in Equation 13. The peak power is equal to:

|  | Peak=maxOFDM(tmax)2 |  |
| --- | --- | --- |

As the OFDM signal is complex the first subplot shows as single unit power complex wave and hence has a peak power of 12, the second subplot has two unit power complex wave forms and therefore has a peak power 22, finally the third subplot has a peak wave form 32. This can be shown mathematically

|  | maxOFDM(t)OFDM*(t)k=0N-1akej2πfktk=0N-1ak*e-j2πfkt |  |
| --- | --- | --- |

Where ak=Xkejθk, if we set ak=1 we obtain the following:

|  | k2=akak*k=0N-1ej2πfkte-j2πfkt |  |
| --- | --- | --- |

Hence confirming the measured peak power for each subplot, however the mean power is determined by

|  | EOFDM(t)OFDM*(t)k=0N-1akej2πfktk=0N-1ak*e-j2πfkt |  |
| --- | --- | --- |

Which is equal to k therefore the peak to mean power ratio is given by:

|  | papr=k2k |  |
| --- | --- | --- |

In this simplified version the papr=k, for subplots one through to three the papr=1,2,3 respectively and hence as the number of active subcarriers increase so does the PAPR. The reason being is that time domain signal is composed of multiple subcarriers where the subcarriers can add constructively or de-constructively, if we take a set of normalised QPSK symbols and show all the possible ways in which the symbols can be summed together we obtain Figure 8 where for a QPSK system there are 24 combinations where the probability of obtaining a peak value is 14. Consider an OFDM symbol with QPSK modulated data it is clear that the mean value would decrease as the number of subcarriers increases as the probability of obtaining a peak value is 14 for every subcarrier therefore the overall probability is equal to 14N. Figure 7 subplot 3 clearly shows that the mean power term is decreasing as k increases where k in this instances ranges from 1 to 3 all other subcarriers have zero amplitude and phase.

Figure 7 - Peak to Mean Power Ratio Example

Figure 8-Constructive and Deconstructive QPSK terms

Figure 9- CDF for varying modulations schemes and bandwidths

# MIMO

In previous tasks the simulation model uses the antenna scheme single input single output (SISO) exploiting time (single carrier) or frequency (multi carrier) domain pre-processing and decoding of transmitted and received data respectively. Employing a MIMO scheme i.e. additional antenna elements are implemented and the eNodeB for downlink or uplink resulting in extra spatial dimension to signal pre-coding and detection. The extra spatial dimension are exploited using space-time processing where the link's performance in terms of one or more possible metrics can be improved. where potential improvements can be aim on the error rate, communication data rate, coverage area and spectral efficiency expressed in terms of bps/Hz/Cell.

There are two configuration of MIMO, firstly Single User-MIMO which defines multiple antenna links between the eNodeB and a single UE. The second case is Multiple User-MIMO which defines multiple antenna links between the eNodeB and multiple UE's. Hence several UE's communicating simultaneously with a common eNodeB using the same frequency and time domain resources.

The benefits of defining multiple antenna radio propagation channels are:

- Diversity Gain

- Array Gain

- Spatial Multiplexing Gain (Transmit Diversity)

Diversity Gain was seen in task 4, where the diversity between multiple channel can be exploited in order to help mitigate the effects of multipath fading, where received signals from each multipath channel are sufficiently uncorrelated. In a SISO model where the channel is quasi-static the received signal is denoted as:

|  | rx=hs+n |  |
| --- | --- | --- |

Where  h = αejθ is the Rayleigh Fading variable , s the modulated symbol and n is the Additive White Gaussian Noise seen at the receiver. The MIMO configuration can be seen in Figure 10

1….NT

1….NR

Figure 10-MU-MIMO Configuration

Defining the model assumption as the following:

- Frequency non-selective fading i.e. the channel is invariant over a block of symbols.

- The propagation times of the signals from NT transmit antenna's to NR receive antenna's is relatively small in comparisons to the block of symbols duration T. Hence for practical simulation purposes the transmit and receive antennas are synchronized.

Therefore the received signals can be denoted as:

|  | rmt=n=1NT hmnsngt+nmt, 0≤t≤T, m=1,2,…,NR |  |
| --- | --- | --- |

Where

rmt is the received signal for m receive antenna's

NT is the number of transmit antenna's

sn is the transmitted block of symbol on the nth transmit antenna

hmn is a complex value representing the multipath channel, hmn is a identical distributed 	and statistically independent from channel to channel

gt is the pulse shape of the interpolation filter

nmt is the noise seen at the mth receiver

T is the duration of the block of symbols

NR is the number of receive antenna's

The signal seen at each receive antenna is consists of a match filter to the pulse gt, where the output is sampled at the end of each symbol interval Ts hence the output at the demodulator corresponds to the mth receiving antenna  and can be represented as:

|  | ym=n=1NT hmnsn+nm , m=1,2,…,NR |  |
| --- | --- | --- |

For mathematical convince the received signal can be represented in matrix for as:

|  | y=Hs+n |  |
| --- | --- | --- |

Where H describes the channel between the transmit and receive antenna in matrix form as:

|  | H= h1,1⋯h1,NT⋮⋱⋮hNR,1⋯hNR,NT |  |
| --- | --- | --- |

There matrix of symbols is defined below where there are s1, s2,…sN symbols split between NT transmit antenna's.

|  | s= s1,1⋯s1,N/NT⋮⋱⋮sNT,1⋯sNT,N/NT |  |
| --- | --- | --- |

The Additive White Gaussian Noise seen at each receive antenna is show below, where noise is added to each symbol received symbol.

|  | n= n1,1⋯n1,N/NT⋮⋱⋮nNR,1⋯nNT,N/NT |  |
| --- | --- | --- |

There received matrix can be shown as the following:

|  | y= h1,1⋯h1,NT⋮⋱⋮hNR,1⋯hNR,NT s1,1⋯s1,N/NT⋮⋱⋮sNT,1⋯sNT,N/NT+n1,1⋯n1,N/NT⋮⋱⋮nNR,1⋯nNT,N/NT |  |
| --- | --- | --- |

Which is the same as writing Equation 23, in matlab these matrix's can be easily manipulated and this implementation of a received signal is computationally efficient. The received signal y at the rth antenna is the summation of all the transmitted signal as where y has NR rows.

Two methods of performing equalisation are shown below where the first method is the same methodology as the previous tasks.

## Zero Forcing

One of the most interesting aspect to MIMO is that the transmitted symbols, on nth transmit antenna's overlap in time and frequency. Assessing time slot one, where there are two transmit and two receive antenna's. The modulated symbol's are grouped in terms of the number of transmit antenna's, considering the case where there are two transmit antenna's and we have symbols s1, s2,s3,s4,…sN-1,sN; the symbols are grouped such that :

|  | Time Slot 1 | Time Slot 2 | Time Slot N |
| --- | --- | --- | --- |
| Antenna 1 | s1 | s3 | sN-1 |
| Antenna 2 | s2 | s4 | sN |

Table 1-MIMO 2x2 Time Slot Example

If we continue the example further to show the received signal for a given antenna we obtain Figure 11 which depicts two transmit signals where each signal undergoes NR channels i.e. in the case two. The signal seen at receive antenna 1 in time slot one is a combination of s1, s2, where s1 and s2 have undergone two different channels, the continuous lines represent h1,1 and h1,2, the dotted lines represent h2,1and h2,2. Therefore s1 and s2 have been effected by h1,1 and h1,2 and the signal seen at the receiver is the summation of both signals

|  | a1=h1,1s1+n1+h1,2s2+n1 |  |
| --- | --- | --- |
|  | a1=h1,1s1+h1,2s2+n1 |  |

NT1

NT2

NR1

NR2

Figure 11-MIMO 2x2 Visualised

Similarly for the second receive antenna can be shown as a2=h2,1s3+h2,2s4+n2, in both cases both symbols undergo separate and statistically independent channels, this is also true for Nrx antenna's.

In previous task the zero force equalisation has been implemented in the following manner:

|  | sn=ynHn |  |
| --- | --- | --- |

However this methodology only supports SISO schemes, in order to equalise for a MIMO scheme the pseudo Inverse for a general matrix of m by n must be considered, where the pseudo inverse matrix is given below

|  | A+=AHA-1AH |  |
| --- | --- | --- |

Where AH denotes the conjugate transpose, therefore we can solve for s as the pseudo inverse matrix H+ meets the requirements H+H=I. Hence equalisation is achieved by:

|  | s=H+y |  |
| --- | --- | --- |

Where H+is determined from a channel estimate, in the following matlab simulation H+ is used and hence no channel estimated has occurred. The ZF equaliser achieves a data rate of NNT hence for a two transmit and receive antenna's the data rate is doubled where at the beginning of section 3, it was stated that one of the benefactors of implementing a MIMO system is the communication data rate. However the ZF equaliser tackles the inference were to two or more symbols occupying a given times slot by trying to null the interference induced by s2 for s1 and vice versa. The cancellation of the channel is shown in the below equation.

|  | HHH-1HH=1h12+h22001h12+h22h1*h2h2*-h1 |  |
| --- | --- | --- |

Essentially this means that when deep fades occur the noise is amplified due to:

|  | Noise Amplification= HHH-1HHn |  |
| --- | --- | --- |

Where n is a matrix of noise shown in Equation 26, as HHH-1HH→0 the noise is amplified thus the zero force equaliser is not the best method of equalisation. In addition the Bit error seen using a zero force equalisation is equal for all m by n MIMO schemes, where the bit error observed is the same a signal transmit signal receive scheme. Hence no diversity gain is achieve, only an improved data rate is obtained. In order to obtain the increase data rates it is vital that a strong multipath is present and hence the channels are uncorrelated. If this is not the case and the channels are close correlated it may not be possible to solve for s due to an imbalance of unknowns. Figure 12 shows the bit error for the zero force equalization for a varied number of transmit antenna.

Figure 12-Zero Force Bit Error rate for various MIMO schemes

## Mean Minimum Square Error

Similar to the previous detector the MMSE linearly combines the received signal to determine an estimate s of the transmitted signal. However in this case the matrix is weighted in accordance to the squared error. The equalisation term in this instance is denoted as:

|  | W=HHH+NoI-1HH |  |
| --- | --- | --- |

Where NoI is the noise variance times by the identity matrix of size NT, this methodology achieves the same data rate performance as the zero force equaliser. However diversity gain can now be achieved as in the event of a deep fade the noise amplification is limited by the term NoI as shown below:

|  | s= HHH+NoI-1HHHs+n |  |
| --- | --- | --- |
|  | s= NoI-1s+HHH+NoI-1HHn |  |

However the symbol s is not returned to its original position due to NoI, this trade off is beneficial as it helps to mitigate noise amplification due to deep fades. The Bit error improvements can be seen in Figure 13.

Figure 13- Zero Force vs. MMSE

## Condition Number

In each of the above matrix the condition number for a given channel matrix H, which provides an indication of the accuracy of linear equation solution. The condition number is a ratio of the largest to smallest singular value in the singular value decomposition of the matrix. A singular value for the channel matrix  H is denoted as the square roots of the Eigen Values of HHH where as stated previously HH is the conjugate transpose. Where H can be denoted in terms of the singular value decomposition:

|  | H=U D VH |  |
| --- | --- | --- |

Where U and V are the unitary matrices i.e. UH= U-1 and D is a diagonal matrix whose elements are the singular values of H. For the zero force equalisation the condition number on average will be worse than that of the minimum mean square error equalisation.

However both cases are not ideal, values of 1 or close to 1 indicate a well condition matrix and hence will provide accurate linear equation solutions. In respect to the BER this means that the channel matrix contains minimal deep fades the close the condition number is to 1 and hence the noise amplification is reduce.

It is possible to achieve an ideal condition number of 1 if the Alamouti scheme is implemented.

## Alamouti

The Alamouti Space Time Block Code introduces transmit diversity, the Alamouti scheme use two transmit antenna's and NR receive antenna's as shown in Figure 14. The diversity order obtained from this implementation is 2NR, where the full rate is achieved. This can be shown if we consider the simple case where NR=1. The Alamouti code is defined as:

|  | G2=s1s2-s2*s1* |  |
| --- | --- | --- |

Where s1 and s2 denote symbols one and two, where  s1 and s2 are transmitted in time slot one from antenna one and two and -s2* and s1* are transmitted in time slot 2 from antenna one and two. Therefore the full spatial code Rs=1 as there are two symbols transmitted in two consecutive time slots, hence the term full rate. If we continue with this simple example then our channel matrix is given as:

|  | H=h1,1h1,2 |  |
| --- | --- | --- |

Where h1,1 and h1,2 are two statistically independent Rayleigh Faded Channels, where the channels are flat fading. There received signal for time slots one and two can be denoted as:

| Time Slot 1 | r1= h1,1 s1+h1,2 s2+n1= h1,1h1,2s1s2+n1 |  |
| --- | --- | --- |
| Time Slot 2 | r2= -h1,1 s2*+h1,2 s1*+n2= h1,1h1,2-s2*s1*+n2 |  |

Which can be expressed in the matrix notation as:

|  | y1y2*=h1,1h1,2h2,1*-h1,1*s1s2+n1n2* |  |
| --- | --- | --- |
|  | y=Hs+n |  |

As stated before the condition of the channel matrix determines the accuracy of the linear equation solution. Due to the Alamouti coding scheme the condition of the channel matrix in the above equation is one for all transmissions. Meaning that encoding the symbols using the Alamouti coding scheme mitigates the effects of the deep fades in the channel when the received symbols in each time slot are combined at the receiver.

1….NR

2T

Figure 14-Alamouti Scheme

Equalisation in this methodology again uses the pseudo inverse matrix, we can characterise the estimated transmitted symbol as:

|  | s=HHHs+HHn |  |
| --- | --- | --- |

Where s in its current states is un-normalised, if we consider the term below:

|  | HHH=h1,12h1,22I |  |
| --- | --- | --- |

We wish to account for the squared magnitude of the channel, in doing so we obtain the following formula:

|  | s=s+HHH-1HHn |  |
| --- | --- | --- |
|  | s=H+*y |  |

The simulated results for a Alamouti scheme is shown in Figure 15, the reason why dual diversity isn't obtained for Alamouti scheme with a signal receiver antenna is that the power is split between both transmit antenna's, hence there is half the energy per bit available.  It is also clear that has the number of receive antenna's increase the available gain through diversity decreases.

Figure 15-Alamoui BER

## Space Time Block Codes

Space time block codes utilise

s1h1

s2h2

s3h3

s4h4

-s1h1

s1h2

-s4h3

s3h4

-s3h1

s4h2

s1h3

s2h4

-s4h1

-s3h2

s1h1

s1h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

s1*h1

T1

T2

T3

T4

T5

T6

T7

s1*h1

s1*h1

s1*h1

s1*h1

T8

# Non-Linear Models

All simulations so far have assumed a  linear transmitter and receiver, however in practise non-linear amplifiers are used as they exhibit greater power efficiency. Hence in environments where power consumptions is limited such as cellular mobile networks, non-linear amplifiers are the preferred choice. Hence as a result the physical components used to implement the RF functions of the transmitter and receiver will  exhibit non-ideal behaviour, causing their characteristics to deviated from the ideal linear case. This is detrimental as a linear amplifier preserves the spectrum of the input signal, whereas the non-linear amplifier generates harmonic's and inter-modulation distortion. Hence the spectrum seen at the amplifier output is spread over a much greater bandwidth than that occupied by the spectrum of the modulator output. Even though harmonic signals are generated at integer multiples of the centre frequency i.e. 2fc, 3fc,… it is not a fundamental problem of the non-linear amplifiers, this is due to the fact that band pass filters are applied and therefore heavily attenuating the harmonic frequencies. Inter-modulation however is a cause of concern as the inter-modulation products are generated such that significant energy is seen in adjacent frequencies causing inter-modulation distortion.

## Inter-modulation

It is important to understand the effects of a non-linear amplifier, hence it is important to explain inter-modulation products in a simplistic manner. As stated prior a device that is non-linear will produce harmonics at integer multiples of the centre frequency 2fc, 3fc,4fc,5fc…, it is also worth noting that no device can be made perfectly and as a result harmonics are always generated even at low levels. The above example defines a single tone waveform however if there are two tones present f1and f2, then the sum and difference of f1and f2 are produced in addition to the harmonics. Assuming f2 is the highest frequency then Table 2 show the inter-modulation products up to the 4th order.

| 1st Order, Even | f1 | f2 | 3rd Order, Odd | 2f1-f2 | 2f2-f1 |
| --- | --- | --- | --- | --- | --- |
| 2nd Order, Even | f1+f2 | f2-f1 | 5th Order, Odd | 3f1-2f2 | 3f2-2f1 |
| 4th Order, Even | 2f2+2f1 | 2f2-2f1 | 7th Order, Odd | 4f1-3f2 | 4f2-3f1 |
| 6th Order, Even | 2f2+2f1 | 2f2-2f1 | 9th Order, Odd | 5f1-4f2 | 5f2-4f1 |

Table 2

The odd order inter-modulation products are close to the fundamental frequencies f1and f2, if we consider a practical example where the input signal to a non-linear amplifier is as follows:

|  | Two Tone Signal=Aej2πf1t+Aej2πf2t2 |  |
| --- | --- | --- |

Where the amplitude is detonated A and is unity, f1 has a frequency of -1 and f2 has a frequency of 2 and t is the time of the waveform. Figure 16 subplot one shows the two tone waveform in the frequency domain, where the signal passes through the non-linear amplifier the output is shown in subplot 2. Where the third, fifth, seventh and ninth odd inter-modulation products can be seen.

Figure 16

Before proceeding further it is important to understand the non-linear amplifier model which determines the signal seen in subplot two of Figure 16.

### AM to AM and AM to PM

It is often a common practice to approximate the measured AM to AM and AM to PM characterise by analytical forms rather than numerical interpolation for obtaining the output amplitude and phase values. A generic model for AM to AM and AM to PM is given below:

| AM to AM | Aμx=αAμx1+βAμx2p12p |  |
| --- | --- | --- |
| AM to PM | θμx=αθμx21+βθμx2 |  |

Where the coefficients αA, βA,αθ,βθ are defined in the following manner:

- αA=1, βA=1, P=3

- αθ=π48, βθ=0.5

These values are usually given in the data sheets of a amplifier in question, or can be can be obtained from data using numerical curve-fitting techniques. The way in which the non-linear model works is that μx is the absolute value of x where x is the input waveform to the non-linear amplifier. The output as a result of the input μx is the amplitude component Aμx and the phase component θμx where the output signal can be denoted as:

|  | NL Amplifier Output=Aejθ |  |
| --- | --- | --- |

As the output is measured for different values of μx, the measurement process is described as a swept power measurement. The characteristics of the amplifier where the input signal is DC is shown in Figure 17, where it can be seen that the amplifier does not have a linear output response. In simulation design it is often common practise to design the amplifier model to exhibit a gain of unity and the amplifier is either characterised by the 1dB compression point or the saturation point. The one dB compression point is where the amplifier output power falls 1dB below the input power and can be seen in Figure 18. From Figure 18 it is fairly clear that the 1dB compression point occurs at unit power, a simpler and clearer representation is that of the gain of the amplifier. Figure 19 shows the gain of the amplifier where the 1db compression point can be clearly seen to occur at unit power.

As stated earlier the points of interest are the odd order inter-modulation product where the third order product is closest to the fundamental frequencies and hence is the major contributor to inter-modulation distortion. It is therefore useful to plot the third order interception point of the amplifier. Firstly if we assess the third order inter-modulation distortion IMD3 from Figure 16. We know that the third order distortion products occur at both 2f1-f2 and 2f2-f1, hence the frequencies of interest are f= -4 and f= 5, if we assume that the tones are equal in magnitude then IMD3 is given as:

|  | IMD3=Po-Po3 |  |
| --- | --- | --- |
|  | IMD3=0--24.1 |  |

Where each power term can be measure directly of the graph and Po is the power of the fundamental frequency and Po3 is the power of the third order inter-modulation product.

|  | OIP3= IMD32+Po |  |
| --- | --- | --- |
|  | OIP3= 24.12+0 |  |

The third order intercept point is therefore given as:

|  | IIP3= OIP3-G |  |
| --- | --- | --- |

Where G is the gain of the amplifier, the third order intercept point can be plotted as a theoretical point, Figure 20 initially shows that the intercept point should cross at 30 dB's i.e. at the point when the input power achieves 30 dB the third order incept point will cross the input power line.  Where the scaling for input power is one to one, however for the third order intercept point it is three to three, thought it is worth noting that certain amplifiers have been shown to deviate from the rule and hence for simulation purposes it is always important to obtain a PA model for the manufacture.

Figure 17-AM to AM and AM to PM response of PA

Figure 18-Input vs. Output of PA

Figure 19- Gain of PA

Figure 20

The back off required is either given from the saturation point to the ideal operating point or from the 1 dB compression point and is usually given in dB. This measure defines the amount of power required to minimise the Error Vector Magnitude, it is clear from Figure 20 that as the power is increases above unit power that the output signal starts to fall away from the input power more rapidly. Hence the output signal is less reprehensive of the input signal, the Error Vector Magnitude is a good indicator for this and is given as:

|  | EVM= i=1Nyi-xi2Ni=1Nxi2N |  |
| --- | --- | --- |

Where x is the signal passed through the amplifier and y is the signal seen at the output of the amplifier. This methodology is essentially putting the Euclidean distance between the input and output signal as a measure of error in the form of a percentage. This allows an engineer to gauge how much effect the amplifier will have on the SNR. Therefore it is often common to plot the EVM vs. the input power.

### Adjacent Channel Power Ratio

An important aspect of any communication system is to determine the power introduced in adjacent channels as a result of inter-modulation distortion. Where the third and 5th order

ACPR = 10log10(power in adjacent channel/ Power in wanted channel)

If we know evaluate the effects on a QPSK signal as we vary the input power from -15dB to 3dB where the signal is 8 times over samples with a initial role off factor of 0.5.

## OFDM

If we know assess an over sampled OFDM system where the power in the adjacent channel is measured in respect to the ACPR. The power in the main lobe from the FFT is -42.3dB, the power in the upper adjacent channel in the -133.89 therefore the ACPR 3.2 dB, if the same information is assessed with welch's power spectrum estimate where the NFFT = 512 the power in the main lobe is now given as -43.2 dB and the power in the adjacent channel is -122.5dB  producing a ACPR of 2.8dB. It can be seen in Figure 21 subplot one that the FFT has fairly good dynamic range

PAPR

Back Off Region

1 dB Compression Point

Average Power

Figure 21

# System Level Model

A cellular system provides wireless coverage over a geographical service area. The service area is divided up into cell sites, where the spectrum for a cell site is divided by the frequency reuse pattern.  In WCDM (3G) and LTE (4G) a frequency reuse pattern of one can be deployed, this report assessing the effects of a frequency reuse pattern N = 1 and N = 3.

Base station

Mobile Phone

Figure 1- Geographical Service Area

Define the System to be simulated

Uniformly distributed pseudorandom coordinates across geographical area.

Compute distance between UE and ENODEB

Compute angle between UE and ENODEB

Determine ENODEB gain for each sector

Determine Path Loss, Including shadow fading

Compute Coupling Loss

Apply 3 dB Handover margin

Compute downlink and uplink RSRP

Compute UE Geometry

Apply Uplink Power Control Algorithm

Compute downlink and uplink SNIR

Check if there are x sets of fully loaded sectors, if not continue to drop UE's

Figure 2 - Flow Chart for Cellular System Simulation

## Define the system to be simulated

The number of sectors in a cell is a simulation parameter, a cell can have the following configurations:

- Omni directional antenna

- Three directional antenna's with 120 degree spacing

- Six directional antenna's with 60 degree spacing

This model uses a tri sectored cell site i.e.  three directional antenna's with 120 degree spacing, where the base station is positioned at the centre of the cell. Each sector can be characterised as a hexagon as shown in Figure 3.

R

L=3R

A

R

R

a

2a

x

Figure 3- Cell Site in terms of Sectors

The radius of the cell site R is defined as the distance between the centre of a sector to the base station, which is equal to the radius of the hexagon representing a sector. For this simulation the cell radius was chosen to be 500m.

The frequency spectrum for a cell site is the total frequency spectrum divided by the reuse factor N = 1, 3,4,7,9,12 , ...etc. The value of N is determined by the following equation:

|  | N= i2+ij+j2 |  |
| --- | --- | --- |

Where i and j are positive integers, for example for the condition where i=j=1, the reuse pattern is determined by Figure 4. The value of N can also be determined by:

|  | N=Total number of channels in a clusterTotal number of channels in a cell |  |
| --- | --- | --- |

Where in the simulation the cluster size is three, where a cluster consists of N cell sites therefore there are 9 channels in a cluster and 3 channels in a cell and hence  N is 3. The reuse distance is given as:

|  | D=3 i2+ij+j2×R=3N×R |  |
| --- | --- | --- |

The spectrum is divided so that each sector in a cell has its own frequency range, the reuse distance is the maximum distance achievable before a frequency is required to be reused. The colours blue, green and red represent the trisected spectrum available and hence Figure 4 clearly demonstrates the reuse pattern used for a reuse factor of 3.

L×j

L×i

D

Figure 4 - Reuse Pattern

As stated prior each sector has a directional antenna, where a sector is represented by 120 degree spacing. The antenna pattern given by 3GPP document TR 36.942 is given as:

|  | Aθ=-min12θθ3dB2,Am, Where-180≤θ≤180 |  |
| --- | --- | --- |

Where θ3dB is the 3 dB beam width which corresponds to 65 degrees, the sectored antennas have a finite front to back ratio Am=20 dB which is the maximum attenuation achievable. The antenna pattern for the sectored cell site is shown in Figure 4.

Figure 5 - Base Station Antenna pattern

The eNodeB antenna gain is determined by the height of the eNodeB and propagation frequency used in the model, the parameters used in 3GPP TR 36.942 is shown in Table 2.

|  | Rural Area | Urban Area |  |
| --- | --- | --- | --- |
|  | 900 MHz | 2000 MHz | 900 MHz |
| eNodeB antenna gain (dBi) (including feeder loss) | 15 | 15 | 12 |
| ENODEB antenna height (m) | 45 | 30 | 30 |

Table 1 - ENODEB Antenna Gain

The UE antenna pattern is assumed to be Omni directional and therefore radiates equally in all direction where the antenna gain is given to be 0 dBi.

The transmit power for the eNodeB is dependent of the BW and is given as 43 dBm for BW ≤5MHz and 46 dBm for BW 10, 15, 20 MHz. The UE's transmit power is determined by the power control algorithm which is discussed in section 3.7, where the maximum transmit power is 24 dBm and the minimum transmit power is -30 dBm.

The Noise figure for the eNodeB is given as 5 dB and the Noise figure for the UE is given as 9 dB, the Noise figure is one of three parameters which determine the effective noise power and hence the signal to noise ratio. The effective noise power is:

|  | ENP=Thermal Noise Density+Noise Figure+10*log10(BW) |  |
| --- | --- | --- |

where the thermal noise is -174 dB, the noise figure is as stated above and the BW is equal to:

|  | BW=Total RB'per UE×fResource Block |  |
| --- | --- | --- |

Where the parameters are given in Table 2.

| E-UTRA BW | #375kHz RENodeB per UE | #UEs |
| --- | --- | --- |
| 5MHz | 4 | 12 |
| 10MHz | 8 | 12 |
| 15MHz | 12 | 12 |
| 20MHz | 16 | 12 |

Table 2 - UE allocation Size

Generically speaking a path loss consists

|  | L=10γlog10d+c |  |
| --- | --- | --- |

Where γ is the path loss exponent, d  is the distance between the transmitter and receiver c  is a constant which accounts for system losses. The path loss exponent is normally defined in the range of 2 to 4, where 2 describes the propagation through free space and 4 is a relatively lossy environment. It is important to note that in indoor environments  where a signal is reflected and attenuated off many surfaces the path loss exponent can reach 6. In environments such as tunnels it is possible that the path loss exponent can reach values below 2 due to the fact that the tunnel is acting as a wave guide.

The 3GPP path loss for a urban environment is given as:

Where Dhb is the base station antenna height in metres, measured from the average rooftop level. R is the (Transmitter-Receiver) separation distance in kilometres and f is the frequency used for the propagation model.

The 3GPP path loss for a rural area environment is given as:

|  | L=69.55+26.16log10f-13.82log10Hb +44.9-6.55log10HblogR-4.78log10f2+18.33log10f-40.94 |  |
| --- | --- | --- |

Where R and f are as per Equation 9, Hb is the base station antenna height above ground in metres.

The path loss for a macro cell is given:

|  | Pathloss Macro=L+LogF |  |
| --- | --- | --- |

Where L is as previously calculated depending on the environment being modelled and LogF is the log normally distributed shadow fading, where the standard deviation for this model is given as 10 dB.

Finally the number of cell sites to be simulated across is required, for this simulation 19 cell sites and therefore 57 sectors are simulated.

## UE positioning

A UE is to be dropped such that it is uniformly distributed across geographical area. In this simulation this is achieved by taking the maximum radius of the geographical area which is equal to:

|  | maxR=ISD×number of tiers-1+ hexagon width |  |
| --- | --- | --- |

Where ISD is the inter site distance shown in Figure 6 and tiers 1,2 and 3 are depicted by the colours blue, green and red. The UE's can be positioned at a uniformly distributed pseudo random distance away for the central base station of length:

|  | r=maxRrand(Number of UE's,1) |  |
| --- | --- | --- |

The angle of the UE is determined by:

|  | θ=2π×rand(Number of UE's,1) |  |
| --- | --- | --- |

The UE coordinates are then determined using simple trigonometry, referring to the below equation and Figure 7:

|  | x=rcosθ, ysinθ |  |
| --- | --- | --- |

Cell Radius R

Inter-site distance 3R

Figure 6 - Cell Sites in terms of tiers

## Transmitter to Receiver separation Distance (T-R)

The (T-R) separation distance can easily be computed for all base stations, where the angle and separation distance is already computed for the central base station. As the coordinates of all base stations are known then the distance r and angle theta can be calculated for the remaining eNodeB's.

A

B

C

rsinθ

rcosθ

r

r= AB2+BC2

Figure 7 - UE angle and distance

## Coupling loss

The coupling loss between the UE and sector site is determined by:

- Path Loss Macro Cell

- UE gain

- eNodeB Gain

The path loss for a macro cell is defined in the initial step " Define the System to be simulated" and calculated once the UE's has been positioned. The angle from the UE is used to determine the eNodeB antenna gain seen at each trisected cell site. The coupling loss between the UE and base station antenna is given as:

|  | coupling loss=maxPathloss Macro-GBS-GUE,MCL |  |
| --- | --- | --- |

Where the Pathloss Macro is as previously calculated, GBS is the gain seen by the UE from all eNodeB's. It is important to note that each base station is a trisected site where three directional antenna's are implemented hence this model will produce 57 antenna gains for the 19 cell sites. The UE gain GUE is constant at 0 dBi, the final value MCL is the minimum coupling loss that can be achieved this takes into account that UE in close proximity to the eNodeB, which will still be limited by the difference between the azimuth and elevation angles of the UE and eNodeB. Essentially meaning that a UE directly underneath the eNodeB antenna has a associated height difference and will not receive full gain from the eNodeB antenna. UE's directly in front of the antenna may also incur a coupling loss due to the near field properties of the propagating wave.

## Handover Margin

A UE is to handover to a new serving cell in the coupling loss between the current serving cell is 3 dB greater than any other cell in the geographical service area. In this model this is shown by initially calculating the ideal coupling loss i.e. removing the effects of log normal shadow fading. A UE is then assigned to a serving sector based on this metric, the initial coupling loss (including log normal shadow fading) used is that of the ideal sector. The initial coupling loss is then compared to all other sectors to see if the 3 dB handover criteria is meet, if so the UE changes to the new serving sector and the system parameters are updated accordingly.

## UE Geometry - Uplink Specific

The uplink coupling loss is assumed be equal to the downlink coupling loss in that snap shot of time. Therefore a UE can determine its geometry from the downlink coupling loss, this is given as:

|  | UE Geometry=coupling loss of none serving sectors dB- coupling loss of serving sector dB |  |
| --- | --- | --- |

This provides a metric which tells the UE which base station it will interfere with the most, the following three scenarios are assed in Figure 8:

- For UE A G=PL Neighbour ≫PL Serving Cell =large dB value

- For UE B G=PL Neighbour=PL Serving Cell =0 dB

- For UE C G=PL Neighbour >PL Serving Cell =+ dB

A negative geometry indicates that the UE requires a handover as the coupling loss between the serving cell and the UE is no longer the minimum coupling loss available. The area between the green lines in each sector on Figure 8 indicate an area in a cell where the interference is at its least due to the low coupling loss between the serving sector and high coupling loss of all other sectors in the cell. However this graphical representation does not take into consideration log normal shadow fading, with log normal shadow fading it is possible for a UE to be in what appears to be a good geometry position from  Figure 8 but have a poor geometry due to shadow fading.

A

B

C

Strong UE Geometry between green lines

Figure 8 - UE Geometry

## Power Control

The purpose of power control is to minimise the transmit power to a sufficient level for the current transmission. If the transmit power is too high then excessive interference is generated, if it is too low then little interference occurs however more retransmission will occur. There are two power control strategies open loop power control and closed loop power control.

### Open Loop Power Control

The open loop strategy does not use feedback from the eNodeB and therefore relies on the UE to determine the transmit power. This is largely achieved by the UE determining the coupling loss between the eNodeb and UE, in practise the is achieved by subtracting higher layer filtered RSRP from the measuring the reference signal power. The down side to open loop power control is that it does not account for interference a UE may cause and differences between the uplink and downlink path loss.

### Closed Loop Power Control

The closed loop power control takes into account feedback from the eNodeB which provides UE specific feedback to adjust the power in order to minimise the effects of interference.  This is especially important when regarding the cell edges as the transmit power is likely to be high due to the large coupling loss meaning the interference generated by that UE is high and thus causing UE in good geometry to have a reduced SINR. Feedback from the eNodeB can be used to reduce the power for UE's in poor geometry and hence improving the overall systems SINR.

### Power Control Algorithm

The power control formula for LTE is given as:

|  | p=minPmax,10log10M+Po+ αPL+∆mcs+f∆i |  |
| --- | --- | --- |

Where:

- Pmax is the maximum transmit power the UE can achieve

- M is the number of assigned resource blocks based on UL grant

- Po is a user specific parameter that is broadcasted with a 1 dB resolution

- α is a cell specific path ls compensation factor α=0, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1 Broadcasted value maybe settable via RRC signaling per UE

- PL is the downlink path los calculated by the UE

- ∆mcs is signalled by the RRC (table entries can be set to zero), MCS is signalled in UL grant

- ∆i is UE specific correction value included in the UL grant. Function f∆i is signalled by higher layers where there are two possibilities accumulated vs. absolute value. (Closed Loop  Power Control)

For the purpose of this simulation a simplified version of the power control algorithm is implemented which is shown in Equation 18.

|  | PPUSCH=minPUEmax,Po+αPL |  |
| --- | --- | --- |

Where the terms Po and α are the variables of focus. Po and α are used to control the average received power spectrum density, and thereby the SINR. The fractional compensation factor alpha enables flexible tradeoffs between capacity and fairness, meaning that the a UE's throughput is reduced in an effort to minimise the effects of interference generated by UE's on the cell edges. By changing the value of alpha in the range α=0, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1 the UE transmit power doesn't increase at the same rate as the path loss except for the case where α=1 where the UE tries to fully compensate for the path loss.

The signal power seen at the eNodeB to achieve a required SINR is effectively the receiver sensitivity, Po which is given as:

|  | Po=effective Noise Power+ Power Margin + Required SNR |  |
| --- | --- | --- |

Where the effective noise power is given in Equation 5, the power margin is set to 3 dB and the required SINR to reach the maximum throughput is given in table A.2 of 3GGP document TR 36.942 which is 23 dB.  Alpha is the variable that implements fractional power when α<1, as stated above the UE's transmit power does not increase at the same rate as the coupling loss for fractional power control. PL is the coupling loss between the UE and eNodeB receiver. Alpha is assessed in this report to show the impact of fractional power control.

## Downlink and Uplink RSRP

The reference signals received power can be computed in following manner:

|  | RSRP=Transmit Power-Coupling Loss Serving Cell |  |
| --- | --- | --- |

This is true for the downlink and uplink where in the case of the uplink the transmit power varies due to the power control algorithm. The interference generated is calculated in a similar manner, except the coupling loss of non serving cells are used as shown below:

|  | Interference=Transmit Power-Coupling Loss Non Serving Cells |  |
| --- | --- | --- |

## Downlink and Uplink SNIR

The signal to noise ratio can be simply calculated and represents how a sector would perform in an isolated scenario and therefore will provide the best throughput rates as no interference is present. The SNR is therefore a good benchmark curve as no simulation results should be better than that of an isolated sector. The signal to noise ratio is given as:

|  | SNR=RSRP-Effective Noise Power |  |
| --- | --- | --- |

The signal to noise ratio plus interference (SINR) is given as:

|  | SINR=RSRP-10log1010(ENP Linear+Interference Linear)10 |  |
| --- | --- | --- |

Where ENP is the effective noise power, the methodology shown above is the same for both downlink and uplink, where the RSRP, effective noise power and interference are subject to uplink or downlink calculations.  For the downlink the interference is generated by the non serving cells transmitting on the same frequency band, whereas for the uplink the inference is generated by the UE's transmitting in sectors with the same frequency band excluding the sector of the wanted signal.

# Results

The simulation results in the following test assume that the E-UTRA BW is 5 MHz and the channel bandwidth is split into resource block of 375 kHz, where there are 4 resource blocks per UE and there are 12 UE's per sector.

## Downlink

#### SNIR

The downlink SNIR is the ratio of received signal power over the effective noise power of the UE plus the interference caused by base stations not assigned to the UE but are on the same frequency band as the UE. Figure 14 shows an urban simulation for N = 1 and N = 3 reuse patterns respectively, referring to Equation 3 it is clear that by increasing the distance between sector with the same frequency significant improves in SNIR are achievable. The CDF shows that for 50%  of the time the SNIR is approximately  ≤2.3 dB for N = 1 where as for the case of N = 3 the SNIR is approximately ≤9 dB. The reason for this becomes apparent as each CDF  is assessed.

#### RSRP

As SNIR is directly related to the received power seen at the UE for the downlink it is important to determine if the configuration has any effect on the RSRP. As the power control is a factor of the bandwidth for the downlink configuration i.e. if the network is configured to ≤5MHz  the transmit power is 43 dBm otherwise for 10, 15, 20 MHz  the transmit power is 46 dBm. It is also logical that the path loss and shadow fading will be comparable for each reuse configuration for the UE's in the central three sectors. Hence Figure 16 shows approximately equal CDF for the RSRP of N = 1 and 3 respectively.

#### Interference

Interference seen in the downlink in a cellular system is a result of a UE receiving unwanted signal from sectors on the same frequency as the UEs assigned sector i.e. co-channels. This is illustrated in Figure 9 where the green arrow indicated the wanted signal from the assigned sector and the red arrows indicated the unwanted signal from all other sectors on the same frequency band. It is clear that if the configuration was for N = 1, then the UE would receive two unwanted signal from the centre cell site causing large amounts of interference due to the close proximity, in addition all other cell site would provide three unwanted signals. The Interference and RSRP are the key factors that determine the SNIR, in the downlink the RSRP and interference is determined by the distance and log normal shadow fading. Figure 18 shows the interference caused by UE in co-channels for N = 1 and 3, where it is clear that more interference will occur in the configuration N = 1.

#### Throughput Per UE

The Throughput Per UE Per Sector shown in Figure 22  that for 50% of the time throughput is ≤1.3 Mbps and ≤3 .2Mbps for N = 1 and 3 respectively. Therefore the throughput per UE is less in the N =1 case, however this doesn't take into consideration the overall throughput achieved for said sector and therefore the spectral efficiency has to be taken into account.

#### Aggregated Throughput

At this point the spectral efficiency is taken into consideration where N = 3 has a third of the available spectrum to be allocated. The Aggregated Throughput for each reuse configuration is shown in Figure 20, where the CDF shows for 50% of the time the Aggregated Throughput is approximately ≤5 Mbps and ≤3.3 Mbps for N = 1 and 3 respectively. The aggregated throughput for N = 3 is significantly less than N = 1, this is due to the spectral efficiency of the network. For N = 1 the entire spectrum is available and therefore for a 5 MHz system the following is true:

|  | N1= 4.5 MHz375 kHz=12RB, N3= 4.5 MHz/3375 kHz=4RB |  |
| --- | --- | --- |

Hence there are 12 resource blocks available per sub frame for N = 1, however there are only 4 resource blocks for N = 1. It is intuitive that each of the central three sectors should obtain the same result for a given reuse pattern as the network is not designed such that it favours a given central tri sector and the UE's are uniformly distributed across the sector. Therefore as the number of drops in a simulation increase the results for a sector will converge.

### Interference Visualised

The UE's represented in the following figures are coloured to indicated the sector they are assigned to, where sector 1  is white, sector 2 is yellow and sector 3 is magenta. The dominate interference caused by a eNodeB can be visually shown, in the case where N = 1 the two remaining antenna's in a cell not assigned to the UE in question induce the most interference, where the dominate antenna will be uniformly distributed between the two antenna's. However it is important to be aware that when simulating this effect in a environment where shadow fading does not occur results in a UE often seeing equal amount of interference from both antennas. Meaning that if the simulator looks for the eNodeB producing the highest interfering signal where in the case sectors two and three are causing interference. If the interference is equal then the matlab function "max" will always choose sector two i.e. the maximum value with the lowest index in the array. The UE's that are subjected to the most interference from a eNodeB are shown in Figure 10 and Figure 11. For the case where N = 1, the eNodeB causing the maximum interference will be uniformly distributed between the remaining unassigned eNodeB's in a sector. The path loss between a UE and unassigned eNodeB is relatively small for Figure 10, where a UE is guaranteed to see a gain equal or greater to the front to back ratio. In the case where N = 3 the front to back ratio would be insignificant due to path loss however for N = 1 it can be inferred from Figure 10 that the UE's will sufferer far greater interference than N = 3.

For N = 3 reuse pattern the interference a UE's suffers is reduced, in this configuration the dominate interfering  antennas for each sector is are clearly shown where the antenna causing the interference are colour matched to the UE.

The UE's that suffer with the worst SNIR  can also be shown for each configuration, Figure 12 shows that the UE's suffering with poor SNIR are on the Cell Edges and in very close proximity to the base station. It might seen counter intuitive that a transmitter close to a receiver would obtain a poor signal quality, however in a trisected site, the receiver is suffers interference from the remaining sectors in the cell. This is not the case for N = 3 as the remaining sectors are on a different frequency band, therefore the SNIR is high close to the base station, this is shown by Figure 13.

Figure 9 - Interference Diagram N = 3

Figure 10 - Interference pattern N = 1

Figure 11 - Interference pattern N = 3

Figure 12 - Minimum SNIR N = 1

Figure 13 - Minimum SNIR N = 3

Figure 14 - Downlink SINR achieved for the first three sectors N = 1 and N = 3

Figure 16 - Downlink RSRP N = 1 and N = 3

Figure 18 - Downlink Interference N = 1 and N = 3

Figure 22 - Downlink Throughput Per UE N = 1 and N = 3

Figure 20 - Downlink Aggregated Throughput N = 1 and N = 3

### The Effects of Shadow Fading

The received signal can be explain in terms of path loss and shadowing, as the UE moves through an urban environment the electrical distance between the eNodeB and UE varies. The simulation uniformly distributes the UE's around the geographical area to simulate varying electrical distance. As the signal propagates through the environment objects such as buildings, trees and any other objects that cause the signal to reflect and attenuate are simulated by applying a log normal shadow fading with a standard deviation that represents the environment. Therefore the electrical signal path may suffer with increased losses, whereas other paths can be less obstructed and have an increased signal strength. When considering multipath the received signal is a summation of multiple paths, assuming two or more paths are present and arrive in phase the constructive interference will occur resulting in an increase in magnitude. However if the phase of the two paths are in anti phase and therefore 180 degrees out of phase the destructive interference will occur causing the magnitude to decrease. These factors are crucial to account for in order to effectively predict the reliability of coverage provided by any UE in a cellular network. Comparing Figure 24 and Figure 25 with Figure 10 and Figure 11 it is clear that UE's suffering with the most interference is much wider spread, where the spreading is determined by the standard deviation set for shadow fading.

Figure 24 - Interference pattern N = 1, Shadow Fading

Figure 25 - Interference pattern N = 3, Shadow Fading

Figure 26 - Minimum SNIR N = 1, Shadow Fading

Figure 27 - Minimum SNIR N = 3, Shadow Fading

## Uplink

### Fractional Power Control

Assessing the nominal configuration where α=1, the power control algorithm is designed the UE will aim to achieve a Reference Signal Power strong enough to obtain full throughput.   The required signal power is shown in Equation 18, Figure 28 clearly shows that the UE is able to achieved the required RSRP 85% of the time in the central sectors. The CDF for the chosen transmit power can be seen in Figure 29 showing a good range of transmit powers within each sector. However this algorithm is an every UE for himself type algorithm, no consideration is made to how much interference is being caused in co-channels. This results in a poor SNIR, high level of interference seen in each sector and a poor average throughput, Figure 29, Figure 30 and Figure 31 demonstrate this respectively. UE's on the sector seems of co-channels cause the most interference, therefore by implementing 0<α<1 we have introduce fractional power control, where the path loss is partial compensated by the power control algorithm. The higher the path loss the smaller the received power is at the eNodeB. This causes the received power from cell edge UE's to diminish, in so doing the interference is reduced.

However by assessing alpha as a standalone variable the inference and signal power will be reduce meaning that alpha should be optimised to work with a required target received power. The received power Po for a value of α can be optimised in the following manner, where SINR is given as:

|  | SINR=P-PL-IN |  |
| --- | --- | --- |

Where P is the transmitted power, PL is the path loss and IN noise pulse interference. The SINR of the system model can then be determined as:

|  | SINR=minPmax-PL-IN,Po+αPL-PL-IN |  |
| --- | --- | --- |

For the case where P=Pmax where Pmax is shown in the first part of the above equation and is given as:

|  | Pmax=Po+αPL |  |
| --- | --- | --- |

The target SINR can be rewritten to show:

|  | SINR0=Po+αPL-PL-IN SINR0=Po+PLα-1-IN |  |
| --- | --- | --- |

The above equation can be rearrange to give the target received power:

|  | Po=PLα-1-SINR0-effective Noise Power + Power Margin Po=PLα-1-SINR0-IN |  |
| --- | --- | --- |

Meaning that alpha and the acceptable noise rise in a cell can be optimised for a given cell. For the case where α=0.8 and there is an acceptable 3 dB noise rise. Various path losses can be simulated in order to find an optimum value for a given value of alpha.

Figure 33 shows a significant increase in average throughput over Figure 32, by changing the value of Po in respect to a change in alpha, it can be seen from Equation 29 that if alpha is reduced such that 0>α<1 then UE's within the specified path loss range from the eNodeB achieve full throughput. Beyond the specified path loss the achieved throughput for a UE drops off rapidly as it approaches the cell edges. This is shown in Figure 34 where the specified path loss is 94 dB and a power margin of 3 dB has been used. Figure 33 shows that a verity of path loss values achieve a similar throughput where it is not possible to obtained a exact throughput by specifying a minim path loss. However it can be assured that a minimum cell edge rate can be achieved for 95% of the users.

Furthermore the power margin above effective noise power can be optimised to accommodate a larger network coverage. Increasing the power margin results in an increase in geographical coverage, however this means more UE's are transmitting at a higher power resulting in an increase in interference. The average throughput for various power margins can be seen in Figure 35 where the higher end power margins result in a decrease in throughput.

### Geometry Power Control

Ideally the power control should be implemented such that the UE asses how much interference it is likely to cause and adjust power accordingly. Therefore it is important understand how much interference the UE will cause, this is shown in Section 3.6. The UE geometry enables the power to be controlled bases on the strongest interfering neighbour. Therefore the transmit power can be worked out by setting the maximum amount of interference a co-channel can cause to Po:

|  | Po=10log10n(Nrise-1) |  |
| --- | --- | --- |

Where n  is the effective noise in linear form and Nrise is the noise rise in linear form. The transmit power is therefore:

|  | P=Po+ MCLstrongest Neighbour |  |
| --- | --- | --- |

Where the min and max limits for the transmit power apply. This means that a UE is limited to a  maximum of noise rise of 10log10Nrise in any co-channel, Nrise is specified by the network provider.  The noise rise has been set such that any co-channel will see maximum of 3 dB rise above noise, Figure 38 shows the observed interference for this configuration. It can be seen that the interference is significantly lower than any of the previous simulations due to the power being limited in respect to noise seen in co-channels.

The noise rise can be optimised similarly to previous optimisation, such that ideal noise rise can be obtained by varying the noise rise over various simulation runs.

It is clear that the simulator is not representing the effects that power control has on the throughput of a system as for the case where all UE transmit at full power all the time the throughput in Figure 42 is significantly better than when power control is implemented in Figure 43.