Task 5 - OFDM

Contents

Contents	2

List of Figures	3

List of Tables	3

1	General	3

1.1	Approvals and Dates	3

1.2	Change Record	3

1.3	Acronyms	3

1.4	References	3

2	OFDM	3

2.1	Sub-Carrier	3

2.2	Transmission	3

2.3	Task 5 Part 1	3

2.3.1	The Effects of Channel Delay	3

2.4	Task 5 Part 2	3

2.5	Task 5 Part 3	3

List of Figures

Figure 1- Channel Impulse Response	3

Figure 2- Minimum Frequency Separation	3

Figure 3-Sub-Carrier Sinusoidal Waves	3

Figure 4-Frequency Shifted Rectangular Filter	3

Figure 5- OFDM - Frequency Domain	3

Figure 6- OFDM Symbols	3

Figure 7-OFDM Modulation	3

Figure 8-OFDM Spectrum Analysis	3

Figure 9-Channel Delayed Spectrum Analysis	3

Figure 10-The Effects of a Time Varying Channel on I and Q Components	3

Figure 11-Cyclic Prefix Implementation	3

Figure 12-Cyclic Extension	3

Figure 13- The Effects of a Time Varying Channel on I and Q Components Where a Cyclic Prefix has been appended to the OFDM Symbol	3

Figure 14-Cyclic Prefix Limitations	3

Figure 15-OFDM with AWGN	3

Figure 16-Two Tap Multipath Channel	3

Figure 17- Spectrum Analysis or Two Tap Multipath Channel	3

Figure 18-Channel with unit amplitudes and random phase	3

Figure 19- Spectrum Analysis or Two Tap Multipath Channel with unit amplitude taps	3

List of Tables

Table 1-System Parameters	3

# General

## Approvals and Dates

|  | Approval Date |
| --- | --- |
| A. E. Jones |  |

## Change Record

| Date | Version |  | Author | Description of Changes |
| --- | --- | --- | --- | --- |
| Date | 0.01 |  |  | Initial Draft |

## Acronyms

| EPC | Evolved Packet Core |
| --- | --- |
| LTE | Long-Term Evolution |

## References

- example reference

# OFDM

In the previous task the Rayleigh fading channel model assumed that the transmit bandwidth was low enough to only induce a single tap in the multipath model. In practise we wish to exploit a large bandwidth to achieve high date rates. However using the current model will result in a considerably large number of taps and therefore obtaining a channel estimate in a noise system is difficult. Figure 1 shows the channel impulse response in a wide bandwidth model, If the signal to noise ratio is small then the taps with smaller magnitude would be harder to distinguish. Hence the channel estimate would not be sufficient enough to correct the I and Q components and the Noise estimate for the LLR would also be insufficient.

Time

Amplitude

Time

Current Implemenation

OFDM Implemenation

Figure 1- Channel Impulse Response

## Sub-Carrier

Implementing an OFDM system is advantageous, OFDM achieves high data rate by splitting the occupied bandwidth into sub-bands with relatively low sub bandwidths ∆f=fN  where N number of subcarriers and f is the occupied bandwidth. The high rate data stream is split into N number of lower rate streams which are transmitted simultaneously. Each sub-band may be independently coded and modulated at a synchronous symbol rate of T=1∆f. If ∆f is chosen to be small enough then the channel frequency response Cf is essential constant across each subcarrier. Hence Cf tends towards a frequency flat model, reducing the number of taps seen in the channel and enabling the channel estimated to obtain a sufficient estimate. Each sub-band is associated with a sinusoidal carrier signal where the signal is denoted as:

|  | skt=cos2πfkt where k=0,1,2,…,N-1 |  |
| --- | --- | --- |

where fk is the frequency of the subcarrier, In order to achieve the minimum spacing required for two orthogonal sinusoidal waves of frequency fk and fj where the duration of the wave is T we require the following form:

|  | 0Tcos2πfkt+θk cos2πfjt+θj dt=0 |  |
| --- | --- | --- |

By integrating between the limits we obtain:

|  | cosθksin2πfk+fjT2πfk+fj+sin2πfk-fjT2πfk-fj+sinθjcos2πfk+fjT-12πfk+fj+cos2πfk-fjT-12πfk-fj=0 |  |
| --- | --- | --- |

by evaluating the equation we know that sinnπ=0 andcos2nπ=1, where n is an integer number.  Assuming  fk+fjT is also an integer number then the equation can be simplified to:

|  | cosθksin2πfk-fjT2πfk-fj+sinθjcos2πfk-fjT-12πfk-fj=0 |  |
| --- | --- | --- |

It can be shown that to achieve frequency orthogonality the minimum separation distance is of the form fk-fj=nT,  n=1,2,…,N-1. If an arbitrary value of θ is chosen ranging from 0 to 2π, then in order for above equation to equal zero then the cosine term must equal one and the sine term must equal zero. Hence to satisfy this requirement 2πfk-fjT=2nπ

|  | 2πfk-fjT=2nπ |  |
| --- | --- | --- |
|  | fk-fj=nT |  |

Therefore for the minimum spacing T=1∆f where the frequency terms are independent  from the phase terms θk and θj. Figure 2 shows the minimum frequency separation where ∆f is 15kHz and the phase is randomly assigned between 0 to 2π, the second subplot show the frequency response of both sinusoidal waves.

Figure 2- Minimum Frequency Separation

## Transmission

For a system with N number of sub-carriers, the symbol rate Ts=TN is reduced by a factor of N relative to the symbol rate on a single carrier system that employs the entire bandwidth fn=1/Ts. Therefore the symbol interval in the OFDM system is given as:

|  | T=NTs |  |
| --- | --- | --- |

Where Ts is the symbol interval for a single sub-carrier, where each subcarrier can be defined as:

|  | 2TXkicos2πfkt-2TXkqsin2πfkt |  |
| --- | --- | --- |
|  | 2TXkej2πfkt |  |

Where Xk = Akejθk is the information signal for the desired modulation scheme, the subcarrier waves from f0 to f4 is shown in Figure 3 where ∆f is again 15kHz. The OFDM symbol is defined over a fixed period T=1∆f and is the summation of all the subcarriers.

Figure 3-Sub-Carrier Sinusoidal Waves

Each subcarrier uses a rectangular filter for the period of the sub-carrier T, This can be defined as a time shifted rectangular filter and denoted below:

|  | k=0N-12TXkejθkej2πfkt×gt-to |  |
| --- | --- | --- |

where gt-to is  the time shifted rectangular filter defined as:

|  | gt-to= A, t-toT 0 , otherwise |  |
| --- | --- | --- |

The Time Shifted Fourier Transform for the rectangular filter with a pulse width of T is defined as:

|  | gt-to= -∞∞gt-toe-jωtdt |  |
| --- | --- | --- |
|  | =-∞∞gt'e-jωt-todt' |  |
|  | =e-jωto-∞∞gt'e-jωtdt' |  |

Setting the integral limits to -T2 to T2

|  | e-jωto1-jωe-jωt limits -T2 to T2 |  |
| --- | --- | --- |
|  | e-jωto1-jωe-jωT2 +1jωejωT2 |  |

Using Euler's form e±jθ=cosθ±jsinθ

|  | e-jωtoejωT2 -e-jωT2 jω |  |
| --- | --- | --- |
|  | 2jsinωT2jωe-jωto |  |
|  | TsinπfkTπfkTe-j2πfkto |  |

Where fk is the frequency of the sub-carrier and therefore the rectangular pulses are modulated onto adjacent carriers that are spaced by exact integer multiples of nfk in the frequency domain. The downlink  ranges from n = -N/2, -N/2+1,..., -1,1, 2,...,N/2 whereas in the uplink n = -N/2, -N/2+1,..., -1,0,1, 2,...,N/2-1, hence in the downlink there is no DC component. It can be seen from Figure 4 that the frequency shifted rectangular pluses do no interfere with each other, Inter symbol interference can be made arbitrarily small through the selection of N. Hence the channel appears to have a fixed frequency response of Cfk k=0,1,…,N-1.

Figure 4-Frequency Shifted Rectangular Filter

For a given modulation scheme there are k number of bits per symbol, where k number of bits are mapped to N Sub Carriers, therefore a given OFDM symbol transmits k×N bits. The OFDM symbol is defined by Equation 1.1 and hence is the summation of all the time domain sequences and the duration of the OFDM symbol is T=1∆f. If we define the number of sub-carriers to be equal to 300 and ∆f=15kHz then the left hand side model of Figure 5 shows that the number of symbols in the frequency domain is equal to the Number of carriers. In order to obtain the modulated OFDM symbol in the time domain the inverse Discrete Fourier Transform is used. Firstly to benefit from an FFT implementation the number of IDFT or DFT points should be an integer power of 2. Secondly the frequency domain is fully occupied and therefore the time domain representation is a T-space model where the synchronisation between the transmitter and receiver would have to be perfect in order to recover the transmitted symbol. In addition a narrow pulse has a wide frequency response due to the large number of frequency components, hence then bandwidth for a simple T space model is large, as bandwidth is expensive this is not ideal. Hence oversampling is required, which is an additional complication in order to achieve and ISI free filter design. Given a bandwidth of 5MHz and an occupied bandwidth of 4.5MHz a Raised cosine filter with a roll off of 54.5 = 0.111 is required.  As shown in task three a low roll off requires a more complex filter design as the number of taps increase and alpha decrease.  Instead the implementation shown on the right hand side of Figure 5 can be utilised to achieve the required results.  Where the number of IDFT points is a integer power of 2 i.e. 512, the outer sub-carriers contain zero energy meaning that frequency domain is not fully occupied and therefore the time domain response is that of an oversampled system.

NSymbols= 300

NSymbols= 300+212 zero Padding

Frequency

Amplitude

Oversample Rate

512/300

T spaced Model

Figure 5- OFDM - Frequency Domain

The OFDM symbol in the frequency domain can be shown in the form of Figure 6 where the modulated symbols are defined as Akejθk, k = 1,2,3,...,K and the zero padding either side of the vector of symbols is defined as 512-k2.

K Symbols

Zero Padding

Figure 6- OFDM Symbols

Therefore the process for OFDM modulation can be seen in Figure 7.

K*N
Data bits

Modulation Mapping
k bits per symbol

exp(j*2*pi*f1*t)

exp(j*2*pi*f2*t)

exp(j*2*pi*f3*t)

exp(j*2*pi*fn*t)

Sub-carrier mapping
n modulation symbols

IFFT

Frequency Domain

Time Domain

Figure 7-OFDM Modulation

## Task 5 Part 1

The system implemented in the following matlab simulations is specified in Table 1.

| Parameter | Value |
| --- | --- |
| Bandwidth | 5MHz |
| Occupied Bandwidth | 4.5MHz |
| Frequency Spacing ∆f | 15kHz |
| Symbol Duration T | 66.667µs |
| Symbol interval in a Single Carrier | Ts=TN= 130.21ns |
| Actual Bandwidth | fn=1Ts=7.68MHz |
| Number Of Sub-Carriers | 300 |
| Number of IFFT points | 512 |
| Oversample Rate | 512300=1.7067 |
| Frame Size | 10ms |
| Number of OFDM Symbols per Frame | 150 |
| Number of Transmitted Symbols | 300*150 = 45000 |
| Number of Transmitted Bits | k*45000, Where k is the number of bits per symbol |

Table 1-System Parameters

Using a QPSK modulation scheme and transmitting a single frame (10ms) the spectrum can be analysis on a OFDM symbol and Frame basis. Figure 8 shows the Welch power spectrum estimate in the second subplot where there is a noticeable difference between the Welch power spectrum estimate and the FFT of a single OFDM symbol. Where the energy seen in a transmitted frame starts to split into FFT bins outside the occupied bandwidth. This is due to the abrupt transitions between OFDM symbols, abrupt changes it time cause an increase in spectral components in the frequency domain, hence the occupied bandwidth of the transmitted frame is greater that that of the transmitted symbol. The frequency guard of 0.5MHz allows for filter design with a sharp roll off to be implemented and hence attenuating any spectral components outside the occupied bandwidth.

Figure 8-OFDM Spectrum Analysis

### The Effects of Channel Delay

Incorporating a channel delay of one time sample period i.e. shifting the OFDM symbol by Ts will result in an unsynchronised OFDM symbol and hence the symbol is no longer periodic. From the previous task it was shown that it was desirable to produce and output signal in the following form:

|  | Yn=Xn*Hn |  |
| --- | --- | --- |

Denoting cyclic convolution in the frequency domain, where Xn is the transmitted signal and  Hn is the channel delay term.  As a propagating wave Xn travels through a delayed channel it is convolved linearly with the channel delay Hn and hence Yn, no longer has the periodic properties for cyclic convolution. Implementing a 40 tap channel where the channel delays are 1Ts, 18Ts, 36Ts, 37Ts producing the FFT's shown in Figure 9. As the time delay is increase the ripple seen on each OFDM symbol is increase accordingly. The magnitude of the wanted signal is no longer unity in the frequency domain, where some of the frequency bins incur significant attenuation. The effects in terms of inter symbol interference can be seen in Figure 10, where the phase and amplitude of the signal as been altered due to the channel. Subplot one clearly shows that the constellation points are almost circulating around the unit power circle, as the delay increase shown in the subsequent subplot the constellation points tend away from the unit power circle.  Hence the constellation points cannot be recovered in the current systems implementation.

Figure 9-Channel Delayed Spectrum Analysis

Figure 10-The Effects of a Time Varying Channel on I and Q Components

If we define a time varying multipath channel the effects a channel has on the received signal yn can be analysed in full. Denoting the multipath channel as:

|  | hk= h0 h1 h2 …hK-1 |  |
| --- | --- | --- |

Setting a random value of hk as the Random Rayleigh Variable and there rest of the channel to zero and hence producing a single tapped channel. The channel impulse response causes a shift it time in respect to the position of the Random Rayleigh Variable. Where the Random Rayleigh Variable is denoted as αejθ. If we donte each subcarrier xn =ej2πfntn as periously shown then the recived signal is in the form.

|  | yn=xn*hk+zn |  |
| --- | --- | --- |
|  | y0=x0.h0+z0 |  |
|  | y1=x0.h0+z0+x1.h1+z1 |  |

This process continues denoting the effect of linear convolution as the signal propagates throught the channel. Where zn is the additive white guassian noise seen at the reciever. It is clear that yn is affected by three things, the signal is unsynchronised and its amplitude and phase componentes are effected by the constructive and destructive summation of the recived multipath signals seen at the reciever. It is worth noting that as the frequency is unalterd by the channel, the orthognality between sub-carries is therefore maintained. The issue arries when the reciever chooses samples which correspond to the delayed terms and hence ISI is introduced as the recieved symbol containes elements of the former OFDM symbols. It is desirable the the transmitted signal xn is periodic in the frequency domain at the point of sampling. In order to achieve this a cyclic extention is introduced where we copy k number of samples of the end of the OFDM symbol to denote the cyclic prefix.

## Task 5 Part 2

In a LTE system the last 36 samples are used as the cylic prefix with a time duration of Ts*k=4.7μs. This provides a time buffer to allow for non-synchronised timing at the reciver and the implementation process can be seen in Figure 11.

OFDM symbol

512 samples

36 samples

Copy end of symbol

CP

Figure 11-Cyclic Prefix Implementation

A visual represenation can be show by taking the cylic prefix of a single sub-carrier i.e sub-carrier +1 where there frequency for said sub-carrier is 15kHz as shown in Figure 12.

Figure 12-Cyclic Extension

The cyclic prefix is however a overhead and thus reduces the data rate, hence the length of the prefix is chosen such that it accounts for the worst case delay spread introduce by the channel.  Implementing a cyclic prefix to the current system we obtain the constellation points shown in Figure 13. Subplots 1 to 3 show that the constellation points circle around the unit power circle, as the time delay increase the separation between constellation point increases. This is because symbol detection in OFDM receivers occur in the frequency domain, if we look at a time shift single in terms of a Fourier Transform Pair we get:

|  | ft-τ= Fωe-jωkτ |  |
| --- | --- | --- |

Where τ denotes the time delay and ωk is the angular frequency of the subcarrier, the Fourier Transform Pair shows that the time delay results in a linear phase ramp in the frequency domain, the change in phase can be computed as a function of frequency using the expression θf=2πfkτ. In subplot 4 the time delay is such that it is greater that the duration of the cyclic prefix mean that the sampled symbol is no longer periodic in the frequency domain and hence the received signal contains aspects of the former OFDM symbol.

Figure 13- The Effects of a Time Varying Channel on I and Q Components Where a Cyclic Prefix has been appended to the OFDM Symbol

Assessing the frequency domain response of the symbol delayed by 36Ts and 37Ts shown in Figure 14, a significant amount of ripple has been introduce when the delay spread is greater that buffer generated by the cyclic prefix resulting in the problem seen in Figure 9. Hence as previously stated the cyclic prefix is designed such that it accounts for the worst case delay spread introduce by the channel.

Figure 14-Cyclic Prefix Limitations

## Task 5 Part 3

In order to account for the change in phase the effects of the channel needs to be accounted for through a channel estimated. This is implemented by introducing reference OFDM symbols in to the transmitted frame. A simple channel estimated can be obtained by setting every 8th OFDM symbol to a reference symbol where the modulated symbols can be set to unity. It is important to note that in practise reference symbols are design such that the exploit certain characteristics of the channel but for the purpose of a simple simulation model the implementation suggested above is sufficient. The receiver has a local replica of the reference symbol therefore the channel estimate can be obtained by:

|  | Hn=YnXn |  |
| --- | --- | --- |

Where Yn is the received reference symbol and Xn is the local replica, the frequency response of the channel can then be decimated back to the T-spaced model which represents the change in phase and amplitude for each subcarrier. Each reference symbol can therefore be used to correct the subsequent OFDM symbols modulated with data until a new reference symbol arrives.  Meaning that the entire frame is not required before equalisation occurs which would undoubtedly cause latency issues. The phase and amplitude aligned signal is therefore denoted as:

|  | Xn=YnHn |  |
| --- | --- | --- |

Where each variable is decimated back to the T-spaced model. Adding AWGN into the system model where the signal to noise ratio is 35dB produces Figure 15 where the channel is of unit value.

Figure 15-OFDM with AWGN

If the model is updated to represent a built up urban environment and hence has no direct line of sight a Random Rayleigh Variable defines the effects of multipath signal after constructive or destructive interference occurs. If the coherent bandwidth of the channel is much greater than the bandwidth of the signal, the channel is said to be frequency flat.  Therefore all frequency components of the signal will experience the same magnitude of fading. Defining ∆f to be sufficiently small ensure that the channel is seen to be frequency flat and therefore allowing each symbol to experience the same magnitude of fading. This helps to prevent the effects of a frequency selective fading where the coherent bandwidth of the channel is smaller than the bandwidth of the signal, resulting in different frequency components of the signal experience uncorrelated fading. Introducing a multipath channel with two taps with a random delay where the maximum delay is equal to the duration of the cyclic prefix. One example of the multipath channel is shown in Figure 16, where ∆t is the time difference between the impulses. As ∆t increases  ∆f decreases where ∆f is shown in subplot 2.

∆f

Figure 16-Two Tap Multipath Channel

The effects on the power spectrum can be seen in  Figure 17 where the channel fading can be seen on the frame and OFDM symbol in subplots one and two respectively.

Figure 17- Spectrum Analysis or Two Tap Multipath Channel

If the channel is designed such that the amplitudes of each tap is constrained to unity the Hn tends towards zero meaning that when deep fades occur, selective subcarriers are of the form:

|  | Xk=YkHk→0 |  |
| --- | --- | --- |

where k is the specific subcarrier in question, this results in certain constellation points being amplified significatnly greater than required as shown in Figure 19. Meaning that certain received symbols are amplified behoned average noise bubble for that SNR value i.e. the stray outlier seen in Figure 19. This can be ammened by implementing a least mean square algorith. Therefore helps to prevent the effects of minimum mean square error as the algorithm will assess the gradient of the data shown in Figure 18 where correction to the weight of the vector is made in the direction of the negative of teh gradient vector.  Hence the symbols currently outside the noise bubble are weighted such that minimum square error is minmised and hence the recieved symbol is more representive of the received symbol for the specified channel conditions.

Figure 18-Channel with unit amplitudes and random phase

Figure 19- Spectrum Analysis or Two Tap Multipath Channel with unit amplitude taps